# Indoor-outdoor thermometer

## Step 1: Make it

### What is it?

Use two micro:bits so you can monitor outdoor temperatures remotely.

### How it works

- This project uses two different programs, one for the outdoor micro:bit which senses the temperature and transmits it on radio group 23.

- The outdoor micro:bit uses its temperature sensor to measure how hot or cold it is.

- It uses radio to send this temperature reading to the indoor micro:bit.

- When the indoor micro:bit receives a temperature reading from outside, it stores it in a variable called outdoorTemp.

- When you press input button A on the indoor micro:bit, it shows its own current temperature reading on its LED display output.

- When you press button B, it shows the temperature reading from outside that it has stored in the outdoorTemp variable.

### What you need

- Two micro:bits

- MakeCode or Python editor

- battery pack

- A waterproof container, such a plastic box

## Step 2: Code it

### Outdoor sensor and transmitter:

### Indoor sensor and receiver:

## Step 3: Improve it

- Make the batteries last longer by having the outdoor micro:bit turn its radio off when it’s not in use and sending temperature readings less often.

- Use variables to track the highest and lowest temperatures recorded.

- Calibrate the readings against another thermometer to see if you need to adjust the micro:bit’s temperature.